How to append vendor specific IE in driver management frames

2018/07/10

### Introduction

This document can teach user, how to append vendor specific IE in Realtek driver management frames by iwpriv (rtwpriv) command.

Please follow those steps to set vendor ie setting.

### Driver setting.

1.	Please enable append vendor ie feature from Makefile

CONFIG_APPEND_VENDOR_IE_ENABLE = n ==> CONFIG_APPEND_VENDOR_IE_ENABLE = y

### Step.

- Insert Realtek driver

Ex: insmod 8821au.ko

- Up interface

Ex: ifconfig wlan0 up

- Use iwpriv tools (or rtwpriv tools) and command “vendor_ie_set” to set vendor ie

Ex: iwpriv wlan0 vendor_ie_set 0,5,dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef

- Can use iwpriv tools (or rtwpriv tools) and command “vendor_ie_get” to get vendor ie setting

- Run hostapd or wpa_supplicant

Note: If want to change vendor IE , after setting , must restart hostapd or wpa_supplicant.

### Command usage

- vendor_ie_set:

iwpriv [interface_name] vendor_ie_set [vendor_ie_number],[vendor_ie_mask],[vendor_ie_context]

interface_name: wifi interface name (ex: wlan0)

vendor_ie_number: want to append vender ie number , can support number 0~4 , totals 5 groups vendor ie.

vendor_ie_mask: which management frame want to add vendor ie.

| Bit | Frame Type | Hex. Number | Device Role |
| --- | --- | --- | --- |
| Bit(0) | beacon | 0x1 | softAP, GO |
| Bit(1) | probe request | 0x2 | STA |
| Bit(2) | probe response | 0x4 | softAP, GO |
| Bit(3) | association request | 0x8 | STA |
| Bit(4) | association response | 0x10 | STA |
| Bit(5) | P2P probe request | 0x20 | P2P device, GC, GO |
| Bit(6) | P2P probe response | 0x40 | P2P device |

Ex1: If you want to append vendor ie to beacon and probe response , vendor_ie_mask will be set "5" , (0x5 = binary 0000 0101)

Ex2: If you want to append vendor ie to P2P probe request and beacon , vendor_ie_mask will be set "21" , (0x21 = binary 0010 0001)

vendor_ie_context: Full vendor ie Hexadecimal context.

ex: [Element ID][Length][OUI][Value]

ex: dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef

[dd] = decimal 221 = Vendor specific Element ID

[33] = decimal 51 bytes = IE Length

[00e04c] = REALEK SEMICONDUCTOR CORP

[0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef] = Value

Setting example: To set group 0 or group 3

iwpriv wlan0 vendor_ie_set 0,5,dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef or iwpriv wlan0 vendor_ie_set 3,3,dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef

- vendor_ie_get:

iwpriv [interface_name] vendor_ie_get [vendor_ie_number]

interface_name: wifi interface name

vendor_ie_number: want check appended vender ie number , can support number 0~4 , totals 5 groups vendor ie.

ex: To get group 0 setting or get group 3 setting

iwpriv wlan0 vendor_ie_get 0 wlan0 vendor_ie_get: Vendor IE num 0 , Mask:5 [Beacon][Probe Resp] Vendor IE: dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef or iwpriv wlan0 vendor_ie_get 3 wlan0 vendor_ie_get: Vendor IE num 3 , Mask:3 [Beacon][Probe Req] Vendor IE: dd3300e04c0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef

- Clear vendor ie setting:

ex: To clear vendor ie number 1 group

iwpriv wlan0 vendor_ie_set 1,0,0

Can use vendor_ie_get to check

iwpriv wlan0 vendor_ie_get 0 wlan0 vendor_ie_get: Vendor IE num 0 , Mask:0 Vendor IE: