Quick Start Guide for ETSI Adaptivity and Carrier Sensing Test

ver. 1.0.1

Contents

Release History	2

1.	Introduction	3

2.	Driver configuration	3

3.	Driver configuration – disable (default setting)	3

4.	Driver configuration – enable for ETSI Adaptivity test	4

5.	Driver configuration – enable for Carrier Sense test	4

- Release History

| 1.0.0 | 2014/09/26 | - Add documentation for ADAPTIVITY_VERSION 7.0, 7.1, 7.2 |
| --- | --- | --- |
| 1.0.1 | 2018/09/28 | - Add periodic idle time declaration. |

- Introduction

Realtek Wi-Fi solutions support both ETSI Adaptivity test and Carrier Sense test. This document describes ways to configure Wi-Fi driver for the corresponding test and simple debug methods.

※ The test item "periodic idle time" can be skipped by a declaration document from chip vendor. Please get the document from chip vendor for Certification Lab reference Before starting test.

※ According to the test requirement, the Wi-Fi performance will drop at interfering environment. Please enable this function only when you really need it.

- Driver configuration

There are 2 configuration arguments in Makefile to control:

- CONFIG_RTW_ADAPTIVITY_EN

  - disable

Disable adaptivity function

  - enable

Enable adaptivity function

- CONFIG_RTW_ADAPTIVITY_MODE

  - normal

For ETSI adaptivity test

  - carrier_sense

For carrier sense test

- Driver configuration – disable (default setting)

To disable Realtek adaptivity function (also the default setting of these 2 configuration arguments):

CONFIG_RTW_ADAPTIVITY_EN = disable

CONFIG_RTW_ADAPTIVITY_MODE = normal

Check the configuration arguments through driver’s proc interface at runtime:

# cat /proc/net/rtl8723bs/wlan0/odm/adaptivity

ADAPTIVITY_VERSION 7.2

RTW_ADAPTIVITY_EN_DISABLE

RTW_ADAPTIVITY_MODE_NORMAL

RTW_ADAPTIVITY_DML_DISABLE

TH_L2H_ini TH_EDCCA_HL_diff IGI_Base ForceEDCCA AdapEn_RSSI IGI_LowerBound

0xf5       7                0x32     0          20          0

- Driver configuration – enable for ETSI Adaptivity test

If your product needs to fit the requirement of ETSI Adaptivity test:

CONFIG_RTW_ADAPTIVITY_EN = enable

CONFIG_RTW_ADAPTIVITY_MODE = normal

Check the configuration arguments through driver’s proc interface at runtime:

# cat /proc/net/rtl8723bs/wlan0/odm/adaptivity

ADAPTIVITY_VERSION 7.1

RTW_ADAPTIVITY_EN_ENABLE

RTW_ADAPTIVITY_MODE_NORMAL

RTW_ADAPTIVITY_DML_DISABLE

TH_L2H_ini TH_EDCCA_HL_diff IGI_Base ForceEDCCA AdapEn_RSSI IGI_LowerBound

0xf5       7                0x32     0          20          0

- Driver configuration – enable for Carrier Sense test

If your product needs to fit the requirement of Carrier Sense test:

CONFIG_RTW_ADAPTIVITY_EN = enable

CONFIG_RTW_ADAPTIVITY_MODE = carrier_sense

Check the configuration arguments through driver’s proc interface at runtime:

# cat /proc/net/rtl8723bs/wlan0/odm/adaptivity

ADAPTIVITY_VERSION 7.1

RTW_ADAPTIVITY_EN_ENABLE

RTW_ADAPTIVITY_MODE_CARRIER_SENSE

RTW_ADAPTIVITY_DML_DISABLE

TH_L2H_ini TH_EDCCA_HL_diff IGI_Base ForceEDCCA AdapEn_RSSI IGI_LowerBound

0xf5       7                0x32     0          20          0