How to set driver debug log level

2016/02/22

- Introduction.

  - The driver’s debug log level is as follows,

enum {

_DRV_NONE_ = 0,

_DRV_ALWAYS_ = 1,

_DRV_ERR_ = 2,

_DRV_WARNING_ = 3,

_DRV_INFO_ = 4,

_DRV_DEBUG_ = 5,

_DRV_MAX_ = 6

};

  - The default log level is “_DRV_INFO_”.

  - The default compiling flag is “CONFIG_RTW_DEBUG”, which is defined in

“Makefile”.

- How to change debug log level during runtime.

  - Use the following command to set different log level.

# echo x > /proc/net/rtl8723bs/log_level

x : is a number mapping to log level in driver.

For example, if you want to change to log level to “_DRV_ERR_”, please execute the command below,

# echo 2 > /proc/net/rtk8723bs/log_level

- How to change debug log level during inserting driver module.

For example for change log level to _DRV_WARNING_,

# insmod 8723bs.ko rtw_drv_log_level=3

- How to disable all debug log level during runtime and inserting driver module.

  - during runtime, please execute the command below,

# echo 0 > /proc/net/rtk8723bs/log_level

  - during inserting driver module , please execute the command below,

# insmod 8723bs.ko rtw_drv_log_level=0

- How to check current log level during runtime

Use the following command to check log level,

# cat /proc/net/rtl8723bs/log_level

drv_log_level:4

_DRV_NONE_ = 0

_DRV_ALWAYS_ = 1

_DRV_ERR_ = 2

_DRV_WARNING_ = 3

+ _DRV_INFO_ = 4

_DRV_DEBUG_ = 5

_DRV_MAX_ = 6

This example shows the current log level is 4 (_DRV_INFO_).

- How to change debug log level during compiling time.

  - In Makefile, please look for “CONFIG_RTW_LOG_LEVEL”, and then you can modify its value to proper debug log level during compiling time.

- How to disable all debug log during compiling time.

  - There are two methods to disable debug log.

    - Method A: in Makefile, please look for “CONFIG_RTW_DEBUG = y”, and then change the “y” to “n”.

    - Method B: also in Makefile, please look for “CONFIG_RTW_LOG_LEVEL”, and then change the default value to 0

- Notes: please change the term “rtl8723bs” to correct chip’s name.