This document had described the way to inform the wpa_supplicant to do the WiFi connection by using the wpa_cli. The wpa_supplicant had supported all kinds of security connections and WPS defined in the 802.11 specification. So, we suggest use the wpa_supplicant to do the WiFi connection rather than the iwconfig wireless tool.

(A) Start WPA_SUPPLICANT

- compile wpa_supplicant

#cd wpa_supplicant #make

- If compile fail like this:

... ... ../src/drivers/driver_nl80211.c:19:31: fatal error: netlink/genl/genl.h: No such file #include <netlink/genl/genl.h> ^ compilation terminated. make: *** [../src/drivers/driver_nl80211.o] Error 1

Try to find which library contains the missing file by:

#sudo apt-file search /netlink/genl/genl.h

If lost libnl, install the library:

# sudo apt-get install libnl-dev or # sudo apt-get install libnl-3-dev

Base on your libnl version to modify .config

For libnl-1.x:

LIBNL=<path to install the libnl> CFLAGS += -I$(LIBNL)/include LIBS += -L$(LIBNL)/lib

For libnl-3.x:

LIBNL=<path to install the libnl> CFLAGS += -I$(LIBNL)/include/libnl3 LIBS += -L$(LIBNL)/lib CONFIG_LIBNL20=y CONFIG_LIBNL32=y

p.s. Version 3.x need add both flag (CONFIG_LIBNL20=y and CONFIG_LIBNL32=y)

For WPA3-SAE:

File: .config CONFIG_TLS=openssl CONFIG_IEEE80211W=y CONFIG_SAE=y

Re-compile wpa_supplicant

# make

- Start wpa_supplicant

  - Run wpa_supplicant in background:

If cfg80211:

# wpa_supplicant –Dnl80211 -iwlan0 -c ../../wpa_0_8.conf –B

Or wireless extensions:

# wpa_supplicant -Dwext -iwlan0 -c ../../wpa_0_8.conf -B

  - Run wpa_supplicant in backqround with debug message (This may affect the performance, only used in debug purpose.)

If cfg80211:

# wpa_supplicant -Dnl80211 -iwlan0 -c ../../wpa_0_8.conf -dd &

Or wireless extension:

# wpa_supplicant -Dwext -iwlan0 -c ../../wpa_0_8.conf -dd &

(B) WPA_CLI commands

- Scaning AP and See Results

# wpa_cli -p/var/run/wpa_supplicant scan # wpa_cli -p/var/run/wpa_supplicant scan_results

- Connect to AP

    - OPEN

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WEP40 with open system

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 1234567890 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WEP40 with shared key mode

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 1234567890 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli –p/var/run/wpa_supplicant set_network 0 auth_alg SHARED # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WEP104 with open system

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 12345678901234567890123456 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WEP104 with shared key mode

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 12345678901234567890123456 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli –p/var/run/wpa_supplicant set_network 0 auth_alg SHARED # wpa_cli -p/var/run/wpa_supplicant select_network 0

- If wep key is ASCII type,use the following cmd:

For WEP40

# wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 '"12345"'

For WEP104

# wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 '"1234567890123"'

- WEP key index is X from 0 to 3, change X for other key index and select it.

# wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_keyX 12345678901234567890123456 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx X

    - TKIP and AES

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt WPA-PSK # wpa_cli -p/var/run/wpa_supplicant set_network 0 psk '"12345678"' # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WPA3-SAE Mode (MFPC=1, MFPR=1)

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt SAE # wpa_cli -p/var/run/wpa_supplicant set_network 0 psk '"12345678"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 ieee80211w 2 # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WPA3-SAE Transition Mode (MFPC=1, MFPR=0)

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 1 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"dlink"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt SAE WPA-PSK # wpa_cli -p/var/run/wpa_supplicant set_network 0 psk '"12345678"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 ieee80211w 1 # wpa_cli -p/var/run/wpa_supplicant select_network 0

- Ad-hoc mode

    - OPEN

# wpa_cli -p/var/run/wpa_supplicant scan # wpa_cli -p/var/run/wpa_supplicant scan_results # wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 2 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"Adhoc_test"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 mode 1 # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 frequency 2412 # wpa_cli -p/var/run/wpa_supplicant select_network 0

#frequency is to set the channel frequency for Ad-hoc master.

    - WEP40

# wpa_cli -p/var/run/wpa_supplicant scan # wpa_cli -p/var/run/wpa_supplicant scan_results # wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 2 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"Adhoc_test"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 mode 1 # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 1234567890 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli -p/var/run/wpa_supplicant set_network 0 frequency 2412 # wpa_cli -p/var/run/wpa_supplicant select_network 0

    - WEP104

# wpa_cli -p/var/run/wpa_supplicant scan # wpa_cli -p/var/run/wpa_supplicant scan_results # wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant ap_scan 2 # wpa_cli -p/var/run/wpa_supplicant add_network # wpa_cli -p/var/run/wpa_supplicant set_network 0 ssid '"Adhoc_test"' # wpa_cli -p/var/run/wpa_supplicant set_network 0 mode 1 # wpa_cli -p/var/run/wpa_supplicant set_network 0 key_mgmt NONE # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_key0 12345678901234567890123456 # wpa_cli -p/var/run/wpa_supplicant set_network 0 wep_tx_keyidx 0 # wpa_cli -p/var/run/wpa_supplicant set_network 0 frequency 2412 # wpa_cli -p/var/run/wpa_supplicant select_network 0

- Save the Current Connection AP configuration file

# wpa_cli -p/var/run/wpa_supplicant save_config

- WPS Connection

- Push Button:

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant wps_pbc any

- Pin Code:

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant wps_pin any 12345670

Or

# wpa_cli -p/var/run/wpa_supplicant remove_network 0 # wpa_cli -p/var/run/wpa_supplicant wps_pin any

- Get Current Status of wpa_supplicant

# wpa_cli -p/var/run/wpa_supplicant status

- Disable current network connection

# wpa_cli -p/var/run/wpa_supplicant disable_network 0

(C) Using WPA_SUPPLICANT by WPA_CLI

(Control interface commands)

- Start wpa_cli control interface:

# wpa_cli

- Commands:

PING

This command can be used to test whether wpa_supplicant is replying to the control interface commands.

The expected reply is PONG if the connection is open and wpa_supplicant is processing commands.

STATUS

Request current status information. The output is a text block with each line in

variable=value format. For example:

bssid=02:00:01:02:03:04

ssid=test network

pairwise_cipher=CCMP

group_cipher=CCMP

key_mgmt=WPA-PSK

wpa_state=COMPLETED

LIST_NETWORKS

List configured networks.

network id / ssid / bssid / flags

0 example network any [CURRENT]

(note: fields are separated with tabs)

SCAN

Request a new BSS scan.

SCAN_RESULTS

Get the latest scan results.

bssid / frequency / signal level / flags / ssid

00:09:5b:95:e0:4e 2412 208 [WPA-PSK-CCMP] jkm private

02:55:24:33:77:a3 2462 187 [WPA-PSK-TKIP] testing

00:09:5b:95:e0:4f 2412 209 jkm guest

(note: fields are separated with tabs)

ADD_NETWORK

Add a new network. This command creates a new network with empty configuration. The new network is

disabled and once it has been configured it can be enabled with ENABLE_NETWORK command. ADD_-

NETWORK returns the network id of the new network or FAIL on failure

SELECT_NETWORK <network id>

Select a network (disable others). Network id can be received from the LIST_NETWORKS command

output.

ENABLE_NETWORK <network id>

Enable a network. Network id can be received from the LIST_NETWORKS command output.

DISABLE_NETWORK <network id>

Disable a network. Network id can be received from the LIST_NETWORKS command output. Special

network id all can be used to disable all network.

.

REMOVE_NETWORK <network id>

Remove a network. Network id can be received from the LIST_NETWORKS command output. Special

network id all can be used to remove all network.

SET_NETWORK <network id> <variable> <value>

Set network variables. Network id can be received from the LIST_NETWORKS command output.

This command uses the same variables and data formats as the configuration file.

• ssid (network name, SSID)

• psk (WPA passphrase or pre-shared key)

• key_mgmt (key management protocol, NONE, WPA-PSK, WPA-EAP)

• proto ( WPA WPA2)

• pairwise ( CCMP TKIP)

• group ( CCMP TKIP WEP40 WEP104)

• wep_key0 ( set wep key for key index 0)

• wep_tx_keyidx ( select wep key index)

• frequency ( Channel frequency in megahertz (MHz) for IBSS )

GET_NETWORK <network id> <variable>

Get network variables. Network id can be received from the LIST_NETWORKS command output.

SAVE_CONFIG

Save the current configuration.

AP_SCAN <ap_scan value>

Change ap_scan value: 0 = no scanning, 1 = wpa_supplicant requests scans and uses scan results to select

the AP, 2 = wpa_supplicant does not use scanning and just requests driver to associate and take care of AP

selection