Quick Start Guide for WPA3

Contents

Introduction	1

1.	WPA3-Peresonal Required for RTK driver	1

2.	WPA3-Enterpris Required for RTK driver (not ready)	2

3.	Start the WPA3-Personal	2

4.	Start the WPA3-Enterprise (not ready)	3

5.	Document revision history	3

- Introduction

The next generation of Wi-Fi® security, bringing new capabilities to enhance Wi-Fi protections in personal and enterprise networks.:

- WPA3-Personal (WPA3-SAE): 
more resilient, password-based authentication even when users choose passwords that fall short of typical complexity recommendations. WPA3 leverages Simultaneous Authentication of Equals (SAE), a secure key establishment protocol between devices, to provide stronger protections for users against password guessing attempts by third parties.

- WPA3-Enterprise (192-bit Mode/Suite B):
offers the equivalent of 192-bit cryptographic strength, providing additional protections for networks transmitting sensitive data, such as government or finance. The 192-bit security suite ensures a consistent combination of cryptographic tools are deployed across WPA3 networks.

- WPA3-Peresonal Required for RTK driver

- Linux Kernel Version

  - Available for WPA3-Personal Station above kernel v4.17. If kernel version below v4.17, you can choose patch kernel or use RTK maintain's hostapd/wpa_supplicant.

  - Available for WPA3-Personal SoftAP above kernel v5.1. If kernel version below v5.1, you can choose patch kernel or use RTK maintain's hostapd/wpa_supplicant.

  - If you use the RTK maintain's hostapd/wpa_supplicant, it can availiable above kernel v3.8.

- Realtek Linux Driver Version

  - Available for WPA3-Personal Station/SoftAP above driver v5.8.

- RTK maintain's hostapd/wpa_supplicant Version

  - For Pure Linux, you have to use version wpa_supplicant_8_O_8x_rtw
above the revision 33457.

  - For Android system, please contact the FAE.

- WPA3-Enterpris Required for RTK driver (not ready)

- Linux Kernel Version

- Realtek Linux Driver Version

- RTK maintain's hostapd/wpa_supplicant Version

- Start the WPA3-Personal

- For further information about wpa_cli and wpa_supplicant, please refer to:
document/wpa_cli_with_wpa_supplicant.pdf.

You have to enable below settings when build wpa_supplicant.

CONFIG_TLS=openssl

CONFIG_IEEE80211W=y

CONFIG_SAE=y

You can scan two kind of WPA3 Acceess Points.

WPA3-SAE mode:
Only WPA3-SAE station can connect.

bssid / frequency / signal level / flags / ssid
00:11:22:33:44:21   2432   -37      [WPA2-SAE-CCMP][WPS][ESS]  WPA3- AP

WPA3-SAE Transition Mode:
WPA2-PSK and WPA3-SAE station can connect.

bssid / frequency / signal level / flags / ssid
00:11:22:33:44:21   2432   -37  [WPA2-PSK+SAE-CCMP][WPS][ESS]  WPA3- AP

You can use the same configuration to connect both Access Point.
The sample configuration as:

ctrl_interface=/var/run/wpa_supplicant

network={

ssid="WPA3-AP"

key_mgmt=SAE

psk="87654321"

ieee80211w=2

}

- For further information about hostapd_cli and hostapd, please refer to:
document/Quick_Start_Guide_for_SoftAP.pdf.

You have to enable below settings when build hostapd.

CONFIG_TLS=openssl

CONFIG_IEEE80211W=y

CONFIG_SAE=y

You can setup the WPA3 SoftAP as:

WPA3-SAE mode:
There are three setting you have to configure as:

auth_algs=3
ieee80211w=2
wpa_key_mgmt=SAE

  - WPA3-SAE Transition Mode:

There are four setting you have to configure as:

auth_algs=3
ieee80211w=1

sae_require_mfp=1
wpa_key_mgmt=SAE WPA-PSK

The sample configuration:

ctrl_interface=/var/run/hostapd

interface=wlan0

driver=nl80211

ssid=WPA3-SAE

channel=1

beacon_int=100

hw_mode=g

ieee80211w=1

auth_algs=3

ignore_broadcast_ssid=0

wpa=2

wpa_passphrase=87654321

wpa_key_mgmt=SAE WPA-PSK

sae_require_mfp=1

wpa_pairwise=CCMP

rsn_pairwise=CCMP

max_num_sta=16

wmm_enabled=1

- Start the WPA3-Enterprise (not ready)

not ready

- Document revision history

| Version | Date YYYY-MM-DD | Remarks |
| --- | --- | --- |
| 1.0 | 2018-05-28 | Initial release |

Support offloading wireless authentication to userspace via NL80211_CMD_EXTERNAL_AUTH

Authentication offload to user space in AP mode

wpa_supplicant_8_O_8.x_rtw_r33457.20190507.tar.gz.