Quick Guide for Wake on WLAN

Date: 2019/2/26

Version: 0.8

Quick Start Guide for Wake on Lan

- Support list:

.) USB interface: 8188EU, 8188CU, 8192DU, 8192EU, 8723BU, 8812AU,

8821AU, 88x2BU, 8188FU, 8723DU

.) SDIO interface: 8189ES, 8189FS, 8723BS, 8703C, 8192ES, 88x2BS,

8821AS, 8703BS, 8723DS, 8723CS.

.)PCI-E interface: 8812AE, 8821AE, 88x2BE, 8821CE, 8723BE.

(2) Requirements of wakeup via in-band and out-band methods:

.) In-band requirements:

- SDIO Interface:

- SDIO host MUST support remote wakeup feature.

- SDIO data1 MUST be wakeup source in the host platform.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

- USB Interface:

- USB host MUST support remote wakeup feature.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

- PCI Interface:

- PCI  host MUST support remote wakeup feature.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

.) Out-band requirements:

- The GPIO of the PLATFORM MUST be wakeup source.

- The platform MUST keep power to WiFi chip in suspend state.

- The platform MSUT work fine between suspend and resume.

- The WIFI module MUST have the GPIO wakeup pin.

(3) Driver Configuration for Wake on WLAN:

.) In-band configuration:

If using SDIO DATA1 pin or USB protocol D+/D- toggle in-band method to wakeup the host, driver need to do is only switch CONFIG_WOWLAN from “n” to “y” in Makefile as Figure 1.

(Figure 1)

.) Out-band configuration:

If using out-band method, driver need to do is modify Makefile and config GPIO. The detail is as following:

- Makefile Configuration:

Switch CONFIG_WOWLAN and CONFIG_GPIO_WAKEUP from “n” to “y” as Figure 2.

(Figure 2)

- GPIO Configuration:

- If use the module package, please use the driver default value. The default value depends on HDK document.

- If there is any customized requirement about  modify WIFI GPIO number, please modiy the value of CONFIG_WAKEUP_GPIO_IDX in Makefile and please contact with RTK technical support team first.

- User could use “proc” subsystem to modify the behavior of WIFI GPIO when receive wakeup up packet in non-suspend state.

  - wowlan_gpio_info to show WIFI wakeup host GPIO number and high_active value:

cat /proc/net/rtlxxxx/wlanX/wowlan_gpio_info

  - modify high_active form 0 to 1 in wowlan_gpio_info:

echo 1 > /proc/net/rtlxxxx/wlanX/wowlan_gpio_info

high_active = 0 means pull low wake. (default)

high_active = 1 means pull high wake.

(Figure 3)

CONFIG_WAKEUP_TYPE:

If the setting of Makefile is CONFIG_WAKEUP_TYPE = 0x7, it means that WOWLAN supports “deauth wake up”, “unicast wake up” and “magic packet wake up”. The detail description is bit0: magic pkt, bit1: unicast and bit2: deauth.

.) Setup the wake up pattern, ONLY support on driver version v5.1.0 or later:

- iwpriv:

iwpriv wlanX wow_set_pattern pattern=[pattern]

Examples:

wake up on any packets sent to MAC 00:E0:4C:01:F0:EE

iwpriv wlanX wow_set_pattern pattern=00:E0:4C:01:F0:EE

- echo pattern into wow_pattern_info:

echo [pattern] > /proc/net/rtl8xxx/wlanx/wow_pattern_info

Examples:

wake up on any packets sent to MAC 00:E0:4C:01:F0:EE

$ echo 00:E0:4C:01:F0:EE > /proc/net/rtl8xxx/wlanx/wow_pattern_info

.) clean wake up patterns, ONLY support on driver version v5.1.0 or later:

- iwpriv:

iwpriv wlanX wow_set_pattern clean

Examples:

wake up on any packet sent to MAC 00:E0:4C:01:F0:EE

- echo clean:

$echo clean> /proc/net/rtl8xxx/wlanx/wow_pattern_info

- Patter Format:

The pattern begins with an 802.3 (Ethernet) header with the correct src/dest MACs base on IPv4. All of the following parameters are need to use HEX format. The more information is as following:

AA:AA:AA:AA:AA:AA:BB:BB:BB:BB:BB:BB:CC:CC:DD:-:-:-:-:-:-:-:-:EE:-:-:FF:FF:FF:FF:GG:GG:GG:GG:HH:HH:II:II

A: Ethernet destination address

B: Ethernet source address

C: Ethernet protocol type

D: IP header VER + Hlen, use: 0x45 (4-is for ver. 4, 5 is for len. 20)

E: IP protocol

F: IP source address (192.168.0.1  C0:A8:00:01)

G: IP destination address (192.168.0.4  C0:A8:00:04)

H: Source port (1024: 04:00)

I: Destination port (1024: 04:00)

(4) The wake up reason table:

The DUT could be waked up by the WIFI chip with the following reasons:

| Reason Value | Description | Note |
| --- | --- | --- |
| 0x01 | Receive pairwise key change packet. |  |
| 0x02 | Receive group key change packet. |  |
| 0x04 | Receive disassociate packet. |  |
| 0x08 | Receive de-auth. Packet. |  |
| 0x10 | AP power off, or could not receive AP’s beacon in a period time |  |
| 0x21 | Receive magic packet. |  |
| 0x22 | Receive unicast packet. | The unicast packet included IP level. |
| 0x23 | Pattern Match | The device could be waked up by specific pattern. |

(5)  wpa_supplicant Configuration for Wake on WLAN:

The configuration file of wpa_supplicant should add “wowlan_triggers=any” when the driver adopt CFG80211 interface in the linux kernel. If there is no “wowlan_triggers=any”, the CFG80211 module will send disconnect command to wifi driver and the wake on WLAN function will fail.

Ex:

ctrl_interface=/var/run/wpa_supplicant #update_config=1 wowlan_triggers=any #connect to open network network={ ssid="SSID" psk="12345678" }

The wpa_supplicant need to restart again after the wifi device/driver remove and insert.

The below command can check the wowlan status of CFG80211:

//get the phy number mapping to wlan interface #iw dev phy#135 Interface wlan1 ifindex 138 type managed phy#134 Interface wlan20 ifindex 137 type managed //get the wowlan status, below is correct #iw phy134 wowlan show WoWLAN is enabled: * wake up on special any trigger //below wowlan status is wrong iw phy134 wowlan show WoWLAN is disabled.