How to Enable Power Saving Functionality in Wi-Fi driver

If you would like to configure various Power Saving Functionin Wi-Fi driver, you can follow as below instructions manual before compiling Wi-Fi driver.

- Main Power Saving Function

- Enable Main Power Saving Function: According to the configuration of IPS, LPS and WOWLAN LPS Function to set various power saving mode.

Set CONFIG_POWER_SAVING to "y" in Makefile as example.

e.g. CONFIG_POWER_SAVING = y

- Disable Main Power Saving Function: Close IPS, LPS and WOWLAN LPS Function.

Set CONFIG_POWER_SAVING to "n" in Makefile as example.

e.g. CONFIG_POWER_SAVING = n

※ The IPS, LPS, WOWLAN LPS funcations are supported after v5.8 driver.

- IPS (Inactive Power Saving) Function

- If the state of Wi-Fi driver is inactive when Wi-Fi connection is not established, enable the IPS function will reduce power consumption.

- The CONFIG_IPS_MODE in Makefile is used for setting IPS mode that will be defined as follow:

    - default: Setting by driver

    - 0: Disable IPS

    - 1: Enable IPS

- LPS (Leisure Power Saving) Function

- If Wi-Fi driver is stayed in idle or low traffic after Wi-Fi connection finished, enable the LPS function will reduce power consumption.

- The CONFIG_LPS_MODE in Makefile is used for setting LPS mode that will be defined as follow:

    - default: Setting by driver

    - 0: Disable LPS

    - 1: Enable LPS

    - 2: Enable LPS with clock gating

- WOWLAN LPS Function

- Enable the WOWLAN LPS function will reduce power consumption when Wi-Fi driver is in WOWLAN mode.

- The CONFIG_WOW_LPS_MODE in Makefile is used for setting WOWLAN LPS mode that will be defined as follow:

    - default: Setting by driver

    - 0: Disable WOWLAN LPS

    - 1: Enable WOWLAN LPS

    - 2: Enable WOWLAN LPS with clock gating